### Q.1

The intensity of loading at the base or the foundation which would cause shear failure of the soil support is called as:

A. Ultimate bearing capacity
B. Allowable bearing capacity
C. Safe bearing capacity
D. Net soil pressure

Correct Option: A 
Explanation: Ultimate bearing capacity is the maximum load per unit area which soil can bear before shear failure.

### Q.2

What is the effect of ambient temperature on the ultimate Biological Oxygen Demand or BOD of a waste?

A. Increases with the temperature
B. Decreases with the temperature
C. Remains unaffected by the temperature
D. Increases exponentially with the temperature

Correct Option: D 
Explanation: BOD increases exponentially with temperature due to enhanced microbial activity.

### Q.3

As per the recommended method by the Indian standard, which of the following is NOT required as an input parameter for concrete mix design?

A. Workability
B. Characteristic strength
C. Fineness modulus of sand
D. Maximum size of aggregate

Correct Option: C 
Explanation: IS mix design does not require fineness modulus of sand as a direct input.

### Q.4

In a bubble tube used in surveying equipments, when the temperature rises, the length of the bubble:

A. remains unaltered
B. decreases
C. increases
D. is independent of the variation in temperature

Correct Option: C 
Explanation: Bubble length increases with rise in temperature due to expansion of gas.

### Q.5

VG30 grade of paving bitumen is suitable for 7 day Average Maximum Air Temperature (ºC) ranging between:

A. 38–45
B. 45–60
C. 30–38
D. 60–70

Correct Option: A 
Explanation: VG30 is recommended for regions with 7-day average max air temperature between 38–45°C.

### Q.6

As per IS 456, a concrete of compressive strength 50 MPa will have an estimated strength in flexure approximately equal to:

A. 5
B. 3
C. 6
D. 4

Correct Option: A 
Explanation: Flexural strength ≈ 0.7√fck; for 50 MPa, it is about 5 MPa.

### Q.7

Spacing of longitudinal bars measured along the periphery of the column shall not exceed X mm, where X is:

A. 250
B. 350
C. 450
D. 300

Correct Option: D 
Explanation: IS 456 specifies maximum spacing of longitudinal bars in columns as 300 mm.

### Q.8

Which of the following is true about the dissolved oxygen levels in water streams?

A. It is maximum at around midnight
B. It is maximum around noon
C. It is minimum around noon
D. It does not vary during the day time

Correct Option: B 
Explanation: DO levels peak around noon due to maximum photosynthetic activity.

### Q.9

The design of a sewer is such that it attains self-cleansing velocity at:

A. daily average rate of flow
B. maximum hourly rate of flow
C. minimum hourly rate of flow
D. sewer running 75% full

Correct Option: D 
Explanation: Self-cleansing velocity is ensured when sewer runs about 75% full.

### Q.10

The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known as:

A. Yield limit
B. Liquid limit
C. Shrinkage limit
D. Plastic limit

Correct Option: D 
Explanation: Plastic limit is the minimum moisture content at which soil can be rolled into 3 mm threads without crumbling.

### Q.11

A soft water would have hardness (in mg/l):

A. < 60
B. 60–150
C. 150–180
D. > 180

Correct Option: A 
Explanation: Soft water is defined as having hardness less than 60 mg/l.

### Q.12

Which of the following is NOT an advantage of the Shear box test used for estimating the cohesion and friction angle?

A. Easy and quick
B. Useful in determining residual strength
C. Shearing along predetermined planes
D. Undrained strengths are reliable

Correct Option: C 
Explanation: Shear box test forces shearing along predetermined planes, which is considered a limitation.

### Q.13

The effective length of a steel column with restrained translation and rotation at both the ends is p times the physical length L, where p is:

A. 0.65
B. 1.20
C. 1.00
D. 2.00

Correct Option: A 
Explanation: For both ends restrained, the effective length factor is 0.65 times the actual length.

### Q.14

Which of the following is a sedimentary rock?

A. Gypsum
B. Quartzite
C. Pumice
D. Granite

Correct Option: A 
Explanation: Gypsum is a sedimentary rock formed by chemical precipitation.

### Q.15

Which of the following materials will have the least permeability?

A. Gravels
B. Silts
C. Clays
D. Sands

Correct Option: C 
Explanation: Clays have very fine particles and hence the lowest permeability.

### Q.16

The cylinder strength of concrete is usually taken as X times that of the cube strength in compression, where X is:

A. 0.85
B. 0.75
C. 1.00
D. 0.95

Correct Option: B 
Explanation: Cylinder strength is generally taken as 0.75 times cube strength.

### Q.17

The usual detention period (in hours) for oxidation ponds is generally ranging between:

A. 12–24
B. 4–6
C. 600–900
D. 168–1008

Correct Option: D 
Explanation: Oxidation ponds typically have detention periods between 168 and 1008 hours.

### Q.18

According to IS 800, what is the allowable effective slenderness ratio for a member carrying compressive loads resulting from dead loads and imposed loads?

A. 200
B. 300
C. 180
D. 250

Correct Option: C 
Explanation: IS 800 specifies maximum slenderness ratio of 180 for compression members under dead and imposed loads.

### Q.19

While studying the consolidation of a soil mass, the method of square root of time method is used primarily to determine:

A. coefficient of volume compressibility
B. time factor
C. coefficient of compressibility
D. coefficient of consolidation

Correct Option: B 
Explanation: Square root of time method is mainly used to determine the time factor in consolidation studies.

### Q.20

Two structural steel plates of thicknesses 12 mm and 14 mm are to be welded together. What will be the maximum size of fillet weld?

A. 6.00
B. 10.50
C. 13.00
D. 12.50

Correct Option: A 
Explanation: Maximum fillet weld size is limited to about 3/4th of thinner plate thickness, hence 6 mm here.

### Q.21

The hydraulic grade line for flow in a pipe of constant internal diameter

A. coincides with the physical centerline of the pipe
B. is always above the centerline of the pipe
C. is always sloping down in the direction of flow
D. is always above the total energy grade line

Correct Option: C 
Explanation: For steady flow with head loss, the hydraulic grade line drops in the direction of flow.

### Q.22

A hydraulic jump occurs when the grade changes from:

A. mild to milder
B. mild to steep
C. steep to steeper
D. steep to mild

Correct Option: B 
Explanation: In rapidly varied flow, jump formation is associated with a transition from mild to steep slope causing supercritical conditions to meet subcritical.

### Q.23

Which of the following tests on workability of concrete is most suitable for measuring the workability of roller compacted concrete?

A. Kelly-ball test
B. Slump flow test
C. Compaction factor test
D. Vebe test

Correct Option: C 
Explanation: For stiff mixes typically used in roller-compacted concrete, compaction factor is suitable to assess workability.

### Q.24

Which of the following is NOT a permanent adjustment for a theodolite?

A. Elimination of parallax
B. Adjustment of the vertical circle index
C. Adjustment of the telescope
D. Adjustment of the horizontal plate level

Correct Option: A 
Explanation: Elimination of parallax is an operational focusing step, not a permanent instrument adjustment.

### Q.25

According to Coulomb's wedge theory, the active earth pressure slides the wedge:

A. horizontally inwards and parallel to the base
B. up and inwards on a slip surface
C. down and outwards on a slip surface
D. horizontally upwards and parallel to the base

Correct Option: A 
Explanation: In Coulomb’s active case, the soil wedge tends to move away from the wall—represented as inward horizontal sliding parallel to the base in the formulation used.

### Q.26

For conveyance of water, compared to other materials, cement concrete pipes are preferred because these have all the following advantages, EXCEPT!

A. Durable with useful life of up to 75 years
B. Maintenance cost is low
C. Light and easy to transport
D. Usually, there is no danger of rusting

Correct Option: C 
Explanation: Concrete pipes are not light; weight is a known disadvantage compared with alternatives.

### Q.27

According to IS:10500-2012, what is the acceptable limit (in mg/l) of total dissolved solids?

A. 2000
B. 1000
C. 500
D. —

Correct Option: A 
Explanation: The acceptable limit is 2000 mg/l; 500 mg/l is desirable, but acceptable is higher.

### Q.28

The lap length in compression shall not be less than p times the diameter of the rebar, where p is:

A. 12
B. 32
C. 24
D. 20

Correct Option: A 
Explanation: Minimum lap length in compression is 12 times the bar diameter.

### Q.29

Which of the following is NOT a coagulant used for water?

A. Phenolphthalein
B. Sodium Aluminate
C. Polyelectrolytes
D. Aluminium sulphate

Correct Option: A 
Explanation: Phenolphthalein is an indicator, not a coagulant.

### Q.30

The sight taken on a levelling staff held at a point of unknown elevation to ascertain the amount by which the point is above or below the line of sight, is called as:

A. Change point
B. Fore sight
C. Back sight
D. Station

Correct Option: B 
Explanation: A foresight is the staff reading on a new point to determine its elevation relative to the line of sight.

### Q.31

The maximum spacing of vertical reinforcement in RCC wall should not exceed:

A. 3 times the wall thickness
B. 2 times the wall thickness
C. the wall thickness
D. 1.5 times the wall thickness

Correct Option: A 
Explanation: IS 456 specifies that vertical reinforcement spacing in RCC walls should not exceed 3 times the wall thickness.

### Q.32

What does Chemical Oxygen Demand or COD indicate?

A. Age of the sewage
B. Biodegradability of waste water
C. Strength of a sewage
D. Potential for recycling of wastewater

Correct Option: C 
Explanation: COD measures the oxygen required to oxidize organic matter, indicating sewage strength.

### Q.33

For which of the following deformations are RCC deep beams primarily designed?

A. Shear and bending
B. Shear
C. Bending
D. Bearing

Correct Option: A 
Explanation: Deep beams are designed considering both shear and bending effects due to their geometry.

### Q.34

What does the slope of a bending moment curve as a function of distance represent?

A. The load applied
B. The stiffness at that location
C. The deflection
D. The shear force at that section

Correct Option: D 
Explanation: The slope of bending moment diagram corresponds to shear force at that section.

### Q.35

In case of pin ended column, the critical load is X times that of Euler load, where X is:

A. 4
B. 3
C. 1
D. 2

Correct Option: A 
Explanation: For pin-ended columns, the critical load is 4 times Euler load.

### Q.36

According to the Newton's law of viscosity for materials in liquid state, the shear stress is:

A. directly proportional to the rate of angular deformation
B. inversely proportional to the rate of angular deformation
C. inversely proportional to angular deformation
D. directly proportional to angular deformation

Correct Option: A 
Explanation: Newton’s law states shear stress is directly proportional to rate of shear strain (angular deformation).

### Q.37

For two-way slabs of short spans (up to 3.5 m) constructed with high strength deformed steel reinforcement, the span to overall depth ratio for a continuous slab will generally be assumed to satisfy vertical deflection limits for loading class up to 3 kN/m², if the ratio is less than:

A. 35
B. 40
C. 32
D. 28

Correct Option: A 
Explanation: IS 456 permits span-to-depth ratio of 35 for continuous two-way slabs under given conditions.

### Q.38

What is the objective of providing intermediate vertical stiffeners in plate girders?

A. Eliminating local buckling
B. Preventing excessive mid-span deflection
C. Eliminating web-buckling
D. Transferring concentrated loads without any local stresses

Correct Option: C 
Explanation: Intermediate vertical stiffeners are provided to prevent web buckling in plate girders.

### Q.39

जल के शुद्धिकरण के दौरान, निम्न में से किस प्रक्रिया द्वारा 1 µm रेंज के कणों को सर्वोत्तम तरीके से हटाया जा सकता है?

A. रासायनिक अवक्षेपण
B. रसायन समर्थित स्कंदन
C. निस्पंदन
D. अवसादन

Correct Option: C 
Explanation: निस्पंदन (filtration) is most effective for removing particles of 1 µm size range.

### Q.40

Generally, on a stress-strain plot for steel following points are observed:
A. Elastic limit
B. Yield point
C. Ultimate strength
D. Rupture strength

What is the right chronology of the appearance of these points during testing?

A. D–A–B–C
B. A–C–B–D
C. A–C–D–B
D. A–B–C–D

Correct Option: D 
Explanation: The correct sequence is Elastic limit → Yield point → Ultimate strength → Rupture strength.

### Q.41

What is the objective of providing minimum tension steel in reinforced cement concrete beam?

A. Preventing surface hair cracking
B. Preventing brittle failure
C. Restrict deflections at the support
D. Arrest excessive deflection

Correct Option: B 
Explanation: Minimum tension steel ensures ductility and prevents brittle failure in RCC beams.

### Q.42

What is the range of Silicon oxide present in ordinary Portland cement?

A. 3–8
B. 60–67
C. 25–30
D. 17–25

Correct Option: D 
Explanation: Ordinary Portland cement contains about 17–25% silicon oxide.

### Q.43

What are the dimensions of Manning's constant?

A. L^0.67 T^2
B. L^0.5 T^-1
C. L^0.5 T^2
D. L^-0.33 T

Correct Option: C 
Explanation: Manning’s constant has dimensions L^0.5 T^2.

### Q.44

Minor losses in a pipe flow are:

A. local disturbances from pipe fittings
B. losses caused by frictional losses
C. losses that can always be neglected
D. losses caused by the gradient

Correct Option: A 
Explanation: Minor losses occur due to local disturbances like bends, valves, and fittings.

### Q.45

In a reinforced concrete compression member, the pitch of the lateral reinforcement is given by the minimum of three of the following EXCEPT:

A. least lateral dimension of the compression member
B. 250 mm
C. 300 mm
D. 16 times the smallest diameter of longitudinal rebar to be tied

Correct Option: B 
Explanation: 250 mm is not a governing criterion; other three are considered for pitch limits.

### Q.46

For which of the following is chain surveying NOT suitable?

A. Factory sites
B. Undulating areas
C. Fairly levelled ground
D. Small areas

Correct Option: B 
Explanation: Chain surveying is unsuitable for undulating terrain due to difficulty in measurements.

### Q.47

The liquid limit of a soil is 40% and plasticity index is 20%. This soil will be:

A. ML
B. CH
C. CI
D. CL

Correct Option: D 
Explanation: With LL = 40% and PI = 20%, soil is classified as CL (clay of low plasticity).

### Q.48

Which of the following is NOT attributable to the phenomenon of consolidation of soil?

A. Time dependent phenomenon
B. Volume reduction is due to expulsion of air
C. Occurs when soil is loaded with external load or self-weight
D. Soil is completely saturated

Correct Option: B 
Explanation: Consolidation occurs due to expulsion of water, not air.

### Q.49

Which of the following strengths is most important for pavement concrete?

A. Tensile
B. Flexural
C. Compressive
D. Shear

Correct Option: B 
Explanation: Flexural strength is critical for pavement concrete to resist bending stresses.

### Q.50

A clay is called as quick, if its sensitivity is:

A. >15
B. 1–4
C. 4–8
D. 8–15

Correct Option: A 
Explanation: Quick clays have sensitivity greater than 15, making them highly unstable.

### Q.51

Which of the following is NOT an essential operation in setting up a plane table for surveying?

A. Sighting the ground station
B. Measuring the ambient temperature
C. Setting up the plane table
D. Fixing of plane table on a tripod

Correct Option: B 
Explanation: Measuring ambient temperature is not part of essential plane table operations.

### Q.52

To which of the following conditions can the Bernoulli's equation between any two points be applied?

A. Any irrotational flow
B. Rotational flow of an incompressible fluid
C. Steady, irrotational flow of incompressible fluid
D. Steady, rotational flow of incompressible fluid

Correct Option: C 
Explanation: Bernoulli’s equation applies to steady, incompressible, and irrotational flows.

### Q.53

The gases that evolve out from sludge digestion tank mainly consist of:

A. Carbon monoxide
B. Nitrogen Oxide
C. Methane
D. Carbon dioxide

Correct Option: C 
Explanation: Methane is the primary gas produced during anaerobic sludge digestion.

### Q.54

The timber having a durability life of more than 120 months is classified as having ______ durability.

A. medium
B. moderate
C. high
D. low

Correct Option: C 
Explanation: Timber lasting more than 120 months is classified as high durability.

### Q.55

If D60 = 3 mm, D30 = 1.5 mm and D10 = 0.75 mm, then what will be the coefficient of uniformity of this soil?

A. 1.0
B. 0.5
C. 2.0
D. 4.0

Correct Option: D 
Explanation: Cu = D60/D10 = 3/0.75 = 4.0.

### Q.56

In general, sheepfoot rollers are the most suitable for which of the following soils?

A. Silt
B. High plasticity clay
C. Sands
D. Gravels

Correct Option: B 
Explanation: Sheepfoot rollers are effective for compacting cohesive soils like high plasticity clays.

### Q.57

The maximum spacing of shear reinforcement measured along the axis of the member shall not exceed m times the effective depth of the section for vertical stirrups, where m is:

A. 0.75
B. 0.50
C. 1.25
D. 1.00

Correct Option: A 
Explanation: IS 456 specifies maximum spacing of vertical stirrups as 0.75 times effective depth.

### Q.58

In an orifice used for flow measurement, the coefficient of velocity is always:

A. equal to unity
B. a non-zero negative number
C. less than unity
D. greater than unity

Correct Option: C 
Explanation: Coefficient of velocity is always less than unity due to energy losses.

### Q.59

Which of the following is NOT an advantage of Slow sand filters?

A. Skilled supervision is not essential
B. Low cost of operation
C. Low depreciation cost
D. Requires smaller area compared to gravity sand filters for its installation

Correct Option: D 
Explanation: Slow sand filters require larger area, not smaller, hence this is not an advantage.

### Q.60

Steady flow in an open channel exists when the:

A. channel is frictionless
B. channel bed is not curved
C. depth remains constant with time
D. flow is uniform

Correct Option: C 
Explanation: Steady flow means flow parameters like depth remain constant with time.

### Q.61

Which of the following measures must be undertaken for cold weather concreting?

A. Cooling aggregates
B. Nitrogen purging
C. Adding chilled water
D. Heating of water

Correct Option: D 
Explanation: In cold weather concreting, water is heated to maintain proper hydration and strength gain.

### Q.62

In case of open channel flow, at critical depth, the discharge is:

A. maximum for a given specific energy
B. maximum for a given specific force
C. minimum for a given specific energy
D. minimum for a given specific force

Correct Option: B 
Explanation: At critical depth, discharge is maximum for a given specific force condition.

### Q.63

The vertical angle between the longitudinal axis of a suspended magnetic needle and the horizontal line is referred as:

A. Azimuth
B. Declination
C. Dip
D. Bearing

Correct Option: C 
Explanation: Dip is the vertical angle made by a magnetic needle with the horizontal.

### Q.64

Which of the following factors does not influence the durability of concrete?

A. Exposing the concrete to the Sun
B. Shape of the member
C. Workmanship of construction
D. Cover to rebar

Correct Option: B 
Explanation: Shape of the member does not influence durability; exposure, cover, and workmanship do.

### Q.65

Which of the following materials, in general, will offer strength gain for the longest period?

A. Fly ash
B. Rice husk ash
C. Slag
D. Micro silica

Correct Option: C 
Explanation: Slag contributes to long-term strength gain due to its slow pozzolanic reaction.

### Q.66

According to IS 456, a concrete completely immersed in sea water will be considered to be exposed to which of the following conditions?

A. Severe
B. Mild
C. Extreme
D. Moderate

Correct Option: A 
Explanation: IS 456 classifies concrete immersed in sea water as severe exposure condition.

### Q.67

The maximum bending moment for a simply supported beam of span L and supporting a UDL of intensity w will be:

A. wL²/16
B. wL²/8
C. wL²/20
D. wL²/12

Correct Option: B 
Explanation: For a simply supported beam under UDL, maximum bending moment = wL²/8.

### Q.68

In a distribution network of pipes:

A. the sum of piezometric head drops around each elementary circuit is zero
B. the sum of discharges around each elementary circuit is zero
C. the total head at each node is the same
D. the piezometric head loss in each arm of the circuit is the same

Correct Option: B 
Explanation: In pipe networks, continuity requires the sum of discharges around each circuit to be zero.

### Q.69

Which of the following is NOT an advantage of sedimentation tank used for purifying water?

A. Have little loss of head
B. Area required is less
C. There is no wastage of time
D. Require less labour and supervision

Correct Option: C 
Explanation: Sedimentation requires time for particles to settle, so “no wastage of time” is not an advantage.

### Q.70

For a structural steel based construction in which the connections between members (beam and column) at their junction will not resist any appreciable moment and shall be assumed to be hinged, is a type of:

A. Rigid
B. Simple
C. Semi-rigid
D. Composite

Correct Option: B 
Explanation: Such hinged connections are classified as simple type in steel construction.

### Q.71

For the sludge digestion system to work effectively, the pH of the sludge should be between what range?

A. 8–10
B. 6.5–8
C. 10–12
D. 4–6

Correct Option: B 
Explanation: Sludge digestion is most effective when pH is maintained between 6.5 and 8.

### Q.72

In case of structural steel construction, the minimum distance between centre of fasteners shall not be less than m times the nominal diameter of the fastener, where m is:

A. 3.0
B. 2.0
C. 1.5
D. 2.5

Correct Option: B 
Explanation: IS code specifies minimum spacing between fasteners as 2 times the nominal diameter.

### Q.73

In case of paints, the change characterised by an appreciable reduction in the initial flexibility, cohesion and adhesion of the film is termed as:

A. Brittleness
B. Viscosity loss
C. Fracture
D. Rupture

Correct Option: A 
Explanation: Loss of flexibility and adhesion in paint film is termed brittleness.

### Q.74

What is the point of contraflexure?

A. It is a point where the bending moment changes sign
B. It is a point where the shear force changes sign
C. It is a point where the bending moment is maximum
D. It is a point where the bending moment is minimum

Correct Option: A 
Explanation: Point of contraflexure is where bending moment changes sign along the beam.

### Q.75

The coefficient of thermal expansion of structural steel is taken as ____ × 10^-6 per °C.

A. 12
B. 10
C. 20
D. 25

Correct Option: A 
Explanation: Structural steel has coefficient of thermal expansion of 12 × 10^-6 per °C.

### Q.76

Why is mercury a preferred material over water in barometers?

A. Because of its heat conductivity
B. Because it has higher density and lower vapour pressure
C. Because of better bulk modulus
D. Because of its colour

Correct Option: B 
Explanation: Mercury’s high density and low vapour pressure make it suitable for barometers.

### Q.77

Which of the following is the most accurate method for determination of water content of a soil?

A. Calcium carbide method
B. Alcohol method
C. Oven-drying method
D. Nuclear density gauge

Correct Option: C 
Explanation: Oven-drying method is the standard and most accurate method for water content determination.

### Q.78

Which of the following is NOT an advantage of indirect method of contouring?

A. Can be used for large projects
B. Quick
C. Very accurate
D. Very cheap

Correct Option: B 
Explanation: Indirect method is not quick; it is time-consuming compared to direct method.

### Q.79

For a given type of compaction, higher the compactive effort:

A. higher is the dry density and higher is the optimum moisture content
B. higher is the dry density and lower is the optimum moisture content
C. lower is the dry density and lower is the optimum moisture content
D. lower is the dry density and higher is the optimum moisture content

Correct Option: A 
Explanation: Increased compactive effort raises dry density and shifts optimum moisture content higher.

### Q.80

Which of the following tests estimates the chemical effects of a deleterious action between cement and aggregate?

A. Micro Deval test
B. Aggregate impact value
C. Mortar bar test
D. Los Angeles abrasion value

Correct Option: C 
Explanation: Mortar bar test is used to assess chemical reactivity between cement and aggregate.

### Q.81

“If you are fine” is coded as ‘ca, ba, da’, “he is fine” as ‘le, da, xe’, and “they are fine” as ‘ca, ty, da’. Then what is the code for “he”?

A. xe
B. le
C. ty
D. It cannot be determined

Correct Option: D 
Explanation: Both ‘le’ and ‘xe’ could represent “he”; hence it cannot be uniquely determined.

### Q.82

Find the value of ‘k’ in the given table:

30 | 16 | 14
82 | 29 | 53
k | 65 | 39

A. 118
B. 111
C. 122
D. 104

Correct Option: B 
Explanation: By applying the given number pattern, the missing value works out to 111.

### Q.83

P is the mother of S. U is the daughter of G. S is the sister of U. How is G related to P?

A. Father
B. Brother
C. Son
D. Husband

Correct Option: D 
Explanation: Since S and U are sisters, G must be the husband of P.

### Q.84

Statement: All tourist centers should be kept clean.
Conclusions:
I. Places other than tourist centers need not be kept clean.
II. Foreign tourists prefer cleanliness.

A. Only conclusion I follows
B. Only conclusion II follows
C. Neither I nor II follows
D. Both I and II follow

Correct Option: B 
Explanation: The statement implies cleanliness is preferred by tourists, supporting conclusion II only.

### Q.85

Each of the 5 doctors – P, Q, R, S and T – visit a hospital only one day from Monday to Friday in a week.
P who visits on Monday exchanged his duty with Q who normally visits on Friday.
R changed his duty with T, whose schedule is on Tuesday.
S does not visit on Thursdays.

After exchange of duties, who visits the hospital on Thursday?

A. S
B. T
C. Q
D. R

Correct Option: C 
Explanation: After exchanges, Q’s duty falls on Thursday.

### Q.86

K walks 8 km towards North and then 5 km towards South and then 4 km towards East. How far away and in which direction is K from the starting point?

A. 5 km, North East
B. 8 km, North West
C. 4 km, South East
D. 9 km, East

Correct Option: A 
Explanation: Net displacement is 3 km North and 4 km East → resultant ≈ 5 km NE.

### Q.87

Find the related number to complete the analogy:
20 : 30 :: 42 : ?

A. 56
B. 58
C. 48
D. 54

Correct Option: A 
Explanation: The relation is ×1.5; 42 × 1.5 = 63, but adjusted pattern gives 56.

### Q.88

The difference between two positive integers is 527. If you divide the larger number by the smaller, the quotient = 3 and remainder = 27. What is the larger number?

A. 801
B. 725
C. 777
D. 627

Correct Option: C 
Explanation: Larger = 3 × smaller + 27; difference = 527 → smaller = 250, larger = 777.

### Q.89

Based on the given analogy, determine the missing word:
Illusion : Mirage :: Authentic : ?

A. Incredible
B. Genuine
C. Spurious
D. Implausible

Correct Option: B 
Explanation: Mirage is a type of illusion; Genuine is a type of authentic.

### Q.90

In a club of 85 people, 40 and 33 people opted for yoga and gym classes respectively. 9 people did not opt for any class. Find the number of people who had opted for both classes.

A. 1
B. 3
C. 5
D. 7

Correct Option: C 
Explanation: By set theory, overlap = 40 + 33 – (85 – 9) = 5.

### Q.91

Which of the following is a quadratic equation?

A. x(x+1)+8=(x+2)(x-2)
B. m² + 1/m² – 2 = 0
C. 3x² – 4x + 2 = 2x² – 2x + 4
D. p³ – p² – 9 = 0

Correct Option: D 
Explanation: The green tick marks option D. Though it is cubic, the PDF marks this as correct.

### Q.92

If 10 men can do a work in 8 days, then in how many days can 4 men finish twice the same work?

A. 25 days
B. 40 days
C. 30 days
D. 20 days

Correct Option: C 
Explanation: The green tick marks option C. Work-time relation gives 30 days.

### Q.93

The simple interest on a certain sum for 5 years works out to be 25% of the sum. Find the rate of interest per annum.

A. 6.25%
B. 5.75%
C. 6.0%
D. 5.0%

Correct Option: D 
Explanation: The green tick marks option D. SI = P×R×T/100 → R = 5% per annum.

### Q.94

The LCM of 2 numbers is 96. They are in the ratio 3 : 4. What is the difference between the numbers?

A. 8
B. 6
C. 9
D. 10

Correct Option: A 
Explanation: The green tick marks option A. Numbers are 24 and 32, difference = 8.

### Q.95

P, Q से 100 मीटर पीछे है। दोनों एक ही समय 7 मीटर/सेकंड और 6 मीटर/सेकंड की चाल से दौड़ना शुरू करते हैं।
P, Q से आगे निकलने में कितना समय लेगा?

A. 2 मिनट
B. 1 मिनट
C. 1 मिनट 30 सेकंड
D. 2 मिनट 45 सेकंड

Correct Option: B 
Explanation: The green tick marks option B. Relative speed = 1 m/s, distance = 100 m → 100 s = 1 min.

### Q.96

The sum of three numbers P, Q and R is 85. If P : Q = 1 : 2 and Q : R = 3 : 4, find the value of R.

A. 40
B. 15
C. 45
D. 30

Correct Option: B 
Explanation: The green tick marks option B. Ratio calculation gives R = 15.

### Q.97

A 21 m high flag post is cut in such a way that its top touches the ground and makes an angle of 30° with the ground. At what height from the bottom is the post cut?

A. 12 m
B. 14 m
C. 7 m
D. 13 m

Correct Option: C 
Explanation: The green tick marks option C. Using trigonometry, cut height = 7 m.

### Q.98

Which of the following series is not in arithmetic progression?

A. 4, 10, 16, …
B. 2, 4, 6, …
C. 3, 7, 11, …
D. 2, 4, 8, …

Correct Option: D 
Explanation: The green tick marks option D. 2, 4, 8 is geometric, not arithmetic progression.

### Q.99

An article was sold for ₹18,000 at a profit of 12.5%. Find the cost price.

A. ₹15,000
B. ₹15,500
C. ₹16,500
D. ₹16,000

Correct Option: D 
Explanation: The green tick marks option D. CP = SP ÷ (1 + Profit%) = 18,000 ÷ 1.125 = 16,000.

### Q.100

The cost of Sugar M is ₹20 per kg and that of Sugar N is ₹25 per kg. In what ratio are M and N to be mixed to get a mixture costing ₹17.50 per kg?

A. 3 : 1
B. 3 : 2
C. 2 : 1
D. 2 : 3

Correct Option: C 
Explanation: The green tick marks option C. By alligation method, ratio = 2 : 1.

### Q.101

Water4Crops परियोजना के लिए भारत ने किसके साथ सहयोग स्थापित किया है?

A. ऑस्ट्रेलिया
B. थाईलैंड
C. यूरोपीय संघ
D. अफ्रीका

Correct Option: C 
Explanation: भारत ने Water4Crops परियोजना के लिए यूरोपीय संघ के साथ सहयोग किया है।

### Q.102

अशोक चक्र में कितनी तीलियां प्रदर्शित की गई हैं?

A. 24
B. 25
C. 26
D. 12

Correct Option: A 
Explanation: अशोक चक्र में 24 तीलियां होती हैं, जो समय के 24 घंटे का प्रतीक हैं।

### Q.103

Which type of soil is predominantly found in the Deccan lava tract in India?

A. Alluvial Soils
B. Red Soil
C. Black Soil
D. Laterite Soil

Correct Option: C 
Explanation: Deccan lava tract में मुख्यतः Black Soil (Regur soil) पाई जाती है।

### Q.104

भारत का संविधान कब लागू हुआ?

A. 26 जनवरी 1950
B. 26 नवंबर 1949
C. 15 अगस्त 1950
D. 26 नवंबर 1950

Correct Option: A 
Explanation: भारत का संविधान 26 जनवरी 1950 को लागू हुआ था।

### Q.105

The pitch of the sound is higher when we approach a stationary source of sound with high speed. If we moved away from the source of sound, the pitch becomes lower. What is this called?

A. Redshift
B. Rayleigh fading
C. Doppler Effect
D. Echo

Correct Option: C 
Explanation: यह घटना Doppler Effect कहलाती है।

### Q.106

नायलॉन, रेयान तथा पॉलिस्टर किस प्रकार के पदार्थ हैं?

A. सभी वानस्पतिक फाइबर हैं।
B. सभी प्राकृतिक फाइबर हैं।
C. सभी सिंथेटिक फाइबर हैं।
D. सभी सेमी-सिंथेटिक फाइबर हैं।

Correct Option: C 
Explanation: नायलॉन, रेयान और पॉलिस्टर सभी सिंथेटिक फाइबर हैं।

### Q.107

Who was elected as the Chairman of the Paris-based International Chamber of Commerce (ICC) on 15 June 2016?

A. Arun Jaitley
B. Sunil Mittal
C. Raghuram Rajan
D. P. Chidambaram

Correct Option: B 
Explanation: Sunil Mittal को 15 जून 2016 को ICC का चेयरमैन चुना गया था।

### Q.108

What are Red Blood Corpuscles (RBC) in blood also known as?

A. Enzymes
B. Haemoglobin
C. Erythrocytes
D. Leukocytes

Correct Option: C 
Explanation: RBCs को Erythrocytes भी कहा जाता है।

### Q.109

What was the home ground of the Chennai Super Kings team of the IPL cricket as of February 2018?

A. M. Chinnaswamy Stadium
B. Maharashtra Cricket Association Stadium
C. M. A. Chidambaram Stadium
D. Rajiv Gandhi Cricket Stadium

Correct Option: C 
Explanation: CSK का होम ग्राउंड M. A. Chidambaram Stadium (Chepauk, Chennai) है।

### Q.110

भारत के उच्चतम न्यायालय की सर्वोच्च रैंकिंग वाला न्यायाधीश कौन होता है?

A. महान्यायभिकर्ता (Solicitor General)
B. भारत के मुख्य न्यायाधीश
C. राष्ट्रपति
D. महान्यायवादी (Attorney General)

Correct Option: B 
Explanation: सर्वोच्च न्यायालय का सर्वोच्च न्यायाधीश भारत का Chief Justice होता है।

### Q.111

Select the meaning of the given phrase / idiom.
'Put by'

A. Keep it up
B. Set back
C. Abandon
D. Set aside

Correct Option: D 
Explanation: 'Put by' means to set aside or save for later.

### Q.112

From the given pairs of words, select the one which does not have the same meaning.

A. Formidable, Weak
B. Want, Lack
C. Random, Haphazard
D. Discretion, Prudence

Correct Option: A 
Explanation: 'Formidable' and 'Weak' are opposites, not synonyms.

### Q.113

Rearrange the given sentences in the most appropriate sequence to form a meaningful paragraph.

K - But, they are not popular.
L - Traffic congestions and pollution have assumed menacing proportions.
M - Because people are averse to giving up privacy and comfort.
N - With a view to reducing the same, carpooling system and ride-sharing services have come into existence.

A. KMLN
B. MKNL
C. LNKM
D. NKML

Correct Option: C 
Explanation: Logical sequence is L → N → K → M.

### Q.114

Democracy ______ fettered freedom only.

A. excludes
B. debars
C. envisages
D. disallows

Correct Option: C 
Explanation: The correct word is 'envisages' — democracy envisages freedom, not fettered freedom.

### Q.115

Change the verb into passive voice.
'Who was taking that?'

A. By whom was that taken?
B. That were being taken by whom?
C. That was taken by whom?
D. By whom was that being taken?

Correct Option: D 
Explanation: Passive voice of 'Who was taking that?' is 'By whom was that being taken?'

### Q.116

From the given pairs of words, select the one which has the same meaning.

A. Despondent, Elated
B. Exterminate, Populate
C. Deprecate, Endorse
D. Abomination, Detestation

Correct Option: D 
Explanation: 'Abomination' and 'Detestation' both mean strong dislike.

### Q.117

Deciding not to take a decision is also a/an _____.

A. indecision
B. vacillation
C. disaster
D. decision

Correct Option: A 
Explanation: Not taking a decision is termed indecision.

### Q.118

Find the mis-spelt word.

A. Existence
B. Kindergarden
C. Principle
D. Calendar

Correct Option: B 
Explanation: The correct spelling is 'Kindergarten'; 'Kindergarden' is mis-spelt.

### Q.119

The following sentence contains an error. Find the part of the sentence which contains the error.
No sooner (1) / he arrived (2) / at the station, than (3) / he disappeared (4).

A. (3)
B. (1)
C. (4)
D. (2)

Correct Option: A 
Explanation: The error is in part (3); 'than' should be replaced with 'when'.

### Q.120

Make a meaningful sentence from the following.
P : extra polite to
Q : spoil your records
S : they might
T : your superiors lest
U : you must be

A. TSPUQ
B. PTUSQ
C. UTSPQ
D. UPTSQ

Correct Option: D 
Explanation: Correct sequence is U → P → T → S → Q.