Q.1 Surveying (Errors in Offsets)

Question:
An offset is measured with an accuracy of 1 in 40. If the scale of plotting is 1 cm = 20 m, find the length of the offset so that the displacement of the point on the paper from both sources of error may not exceed 0.25 mm.

Options:
A) 102 m
B) 252 m
C) 202 m
D) 152 m

Answer: Option C (202 m)

Explanation:
To find the limiting length of the offset, we assume the total error on the ground is the resultant of the plotting error and the measurement error.
Scale: 1 cm = 20 m.
Max allowed displacement on paper: 0.25 mm.
Max allowed displacement on ground: 0.25 mm×(20 m/10 mm)×100=0.5 m.
Measurement Accuracy: 1 in 40, so error = Length (l)40.
Condition: Assuming the plotting error and measurement error contribute equally to the total permissible error (or using the root sum square method where individual components are equal to Limit/2):

Measurement Error=Total Error2
l40=0.52
l=202 m

Q.2 Steel Structures (Stiffeners)

Question:
Which of the following pair of stiffeners with their definition is not correctly matched?

Options:
A) Bearing stiffener: To prevent local crushing of the web due to concentrated loading.
B) Load carrying stiffener: To transmit tensile forces applied to a web through a flange.
C) Diagonal stiffener: To provide local reinforcement to a web under shear and bearing.
D) Intermediate transverse web stiffener: To improve the buckling strength of a slender web due to shear.

Answer: Option B

Explanation:
This is the incorrect match. Load carrying stiffeners are designed to transmit forces to the web, but they are typically used for compressive forces (concentrated loads) to prevent local buckling or crushing of the web, not specifically for tensile forces. Tensile forces generally do not cause the instability issues (buckling) that stiffeners are meant to counteract.

Q.3 Soil Mechanics (Phase Relationships)

Question:
The relationship between void ratio (e), percentage of air voids (na) and degree of saturation (S) is given by:

Options:
A) na=1-S1+e
B) na=e(1-S)1+e
C) na=e(1-S)1-e
D) na=e(1+S)1+e

Answer: Option B

Explanation:
Percentage of air voids (na): Volume of air / Total Volume (Va/V).
Air content (ac): Volume of air / Volume of voids (Va/Vv) = 1-S (where S is degree of saturation).
Porosity (n): Volume of voids / Total Volume (Vv/V) = e1+e.
Derivation:

na=VaV=VaVv×VvV=ac×n
na=(1-S)×e1+e=e(1-S)1+e

Q.4 Environmental Engineering (Filtration)

Question:
As per IS 11401 (part 2): 1990, the recommended value of filtration rate for slow sand filters in the rural water supply is:

Options:
A) 0.4–0.5 m/h
B) 0.1–0.2 m/h
C) 0.25–0.3 m/h
D) 0.3–0.43 m/h

Answer: Option B

Explanation:
Slow sand filters are designed to operate at low filtration rates to promote the growth of the Schmutzdecke (biological layer) which is essential for removing pathogens and impurities. The standard rate is 0.1 to 0.2 m/h (approx. 100-200 liters per hour per square meter). Rapid sand filters, by contrast, operate at much higher rates (e.g., 3-6 m/h).

Q.5 Hydrology (Runoff Empirical Formulas)

Question:
Which of the following relationship is proposed by Inglis to compute runoff (R) in centimeters from the precipitation (P) data in centimeters for Deccan Plateau of India?

Options:
A) R=0.85P+30.5
B) R=0.85P-30.5
C) R=1254P(P+17.8)
D) R=1254P(P-17.8)

Answer: Option D

Explanation:
Inglis derived two formulas for the Maharashtra region:
Ghat Regions: R=0.85P-30.5
Non-Ghat Regions (Deccan Plateau): R=P(P-17.8)254
Since the question specifies the Deccan Plateau, the correct formula is the one shown in Option D.

Q.6 Highway Materials (Bitumen Testing)

Question:
The Pensky-Marten test is used for testing:

Options:
A) Penetration
B) Fire point
C) Viscosity of bitumen
D) Ductility

Answer: Option B

Explanation:
The Pensky-Martens apparatus (closed cup tester) is the standard equipment used to determine the Flash and Fire points of bituminous materials.
Penetration is measured by a Penetrometer.
Viscosity is measured by a Viscometer (e.g., Tar Viscometer).
Ductility is measured using a Ductility testing machine.

Q.7 Highway Materials (Bitumen Properties)

Question:
Which is the property of Bitumen?

Options:
A) It becomes soft at 30∘C to 100∘C.
B) It is soluble in water.
C) It cannot resist acid well.
D) Its components are Carbon, Sulphur and Phosphorous.

Answer: Option A

Explanation:
Temperature Susceptibility: Bitumen is thermoplastic; it softens as temperature increases. The softening point typically falls between 30∘C and 100∘C depending on the grade.
Solubility: Bitumen is insoluble in water (hydrophobic) but soluble in organic solvents like carbon disulfide (CS2) and carbon tetrachloride (CCl4).
Chemical Nature: It is a hydrocarbon (mainly Carbon and Hydrogen), not primarily Sulphur/Phosphorous, and it is generally resistant to acids.

Q.8 Highway Engineering (Geometric Design)

Question:
The purpose of providing camber in highways is to:

Options:
A) Provide stability of vehicle against skidding
B) Drain off the rainwater as quickly as possible
C) Promote centrifugal force caused by vehicle
D) Provide resistance to friction caused by tyres

Answer: Option B

Explanation:
Camber (Cross slope) is the transverse slope provided to the road surface. Its primary function is to drain rainwater from the pavement surface to the edges. This prevents water from percolating into the subgrade (which would weaken the road foundation) and prevents hydroplaning/skidding due to standing water.

Q.9 Irrigation Engineering (Water Quality)

Question:
The soluble salt present in irrigation water when expressed in parts per million (PPM), the amount in excess of ___ is harmful to plants.

Options:
A) 800 PPM
B) 700 PPM
C) 600 PPM
D) 500 PPM

Answer: Option B

Explanation:
High salinity in irrigation water increases the osmotic pressure of the soil solution, making it difficult for plants to extract water. Generally, a salt concentration exceeding 700 PPM (approx. 2000μS/cm electrical conductivity) is considered injurious to most crops.

Q.10 Structural Analysis (Deflection)

Question:
Choose one of the assumptions for Macaulay's method of slope and deflection?

Options:
A) The deflections are large.
B) The material is isotropic.
C) dv/dx is negligible compared to unity.
D) This theory holds good if the deflections are of the order of less than 1/12th the depth of beam.

Answer: Option B

Explanation:
Macaulay's method uses the governing differential equation of the elastic curve (EId2ydx2=M), which relies on the Bernoulli-Euler beam theory. A fundamental assumption of this theory is that the beam material is homogeneous and isotropic (having the same elastic properties in all directions). Other assumptions include small deflections and that plane sections remain plane during bending.

Q.11 Fluid Mechanics (Pumps)

Question:
Which of the following statements is NOT true in the case of centrifugal pump compared to reciprocating pump?

Options:
A) The discharge is continuous and smooth.
B) Efficiency is high as compared to reciprocating pump.
C) The cost of centrifugal pump is nearly four time the cost of reciprocating pump.
D) It is used to lift highly viscous liquids.

Answer: Option 3

Explanation:
The statement in Option 3 is false, making it the correct answer.
Cost: Centrifugal pumps are generally less expensive than reciprocating pumps for comparable capacity because they have fewer moving parts and a simpler construction.
Viscosity: Centrifugal pumps are not suitable for highly viscous liquids (efficiency drops drastically), whereas reciprocating pumps handle them better. However, Option 3 is a more definitive factual error regarding cost.

Q.12 Concrete Technology (Mix Design)

Question:
Which of these should be the maximum total quantity of dry aggregates by mass per 50 kg of cement required, to be taken as the sum of the individual masses of fine and coarse aggregates for M15 grade concrete?

Options:
A) 170 kg
B) 250 kg
C) 330 kg
D) 480 kg

Answer: Option 3

Explanation:
According to IS 456:2000 (Table 9) for nominal mix concrete:
M15 Grade (1:2:4): The total mass of dry aggregates (Fine + Coarse) per 50 kg of cement is specified as 330 kg.
This assumes a water-cement ratio of roughly 0.5 to 0.6.

Q.13 Steel Structures (Buckling)

Question:
Maximum values of effective slenderness ratios for compression flange of a steel beam against lateral torsional buckling as per IS 800-2007 is:

Options:
A) 250
B) 180
C) 400
D) 300

Answer: Option 4

Explanation:
As per IS 800:2007 (Table 3), the maximum effective slenderness ratio (KL/r) for:
A member normally acting as a tie... : 350.
Compression flange of a beam against lateral torsional buckling: 300.
Members carrying compressive loads resulting from dead loads and imposed loads: 180.

Q.14 Irrigation (Crop Water Requirement)

Question:
What is the crop water requirement (mm) for maize in India?

Options:
A) 1200–2200
B) 1500–2500
C) 700–1300
D) 500–800

Answer: Option 4

Explanation:
Maize is a crop that requires a moderate amount of water. The typical water requirement for maize during its base period is 500 mm to 800 mm.
Rice: 900–2500 mm.
Sugarcane: 1500–2500 mm.
Cotton: 700–1300 mm.

Q.15 Fluid Mechanics (Pumps)

Question:
Identify the true statement for centrifugal pump when compared with reciprocating pump?

Options:
A) The torque is not uniform.
B) Complicated construction.
C) Maintenance charges are high.
D) Installation is easy.

Answer: Option 4

Explanation:
Centrifugal pumps are compact, have few moving parts, and require less foundation work, making installation easy and maintenance costs low.
Reciprocating pumps have a pulsating discharge (non-uniform torque), are heavy, complicated, and require more maintenance.

Q.16 Soil Mechanics (Shear Strength)

Question:
Select the correct option from the following statements related to shear strength of soil.
According to Mohr's theory, the failure envelope is a straight line.
The Mohr circle for unconfined compression test passes through the origin.

Options:
A) Statement 1 is true and 2 is false
B) Statement 1 is false and 2 is true
C) Both statements are true
D) Both statements are false

Answer: Option 3

Explanation:
Statement 1 (True): Mohr-Coulomb failure theory approximates the failure envelope as a straight line equation: τ=c+σtan⁡ϕ.
Statement 2 (True): In an Unconfined Compression Test (UCS), the confining pressure (σ3) is zero. Since the Mohr circle is plotted between σ1 and σ3, the circle starts at σ3=0 (the origin) and ends at σ1=qu.

Q.17 Surveying (Tacheometry)

Question:
In tacheometric survey, the additive constant is given by:
(Where, f = focal length of objective, d = distance between optical centre and vertical axis of instrument)

Options:
A) f×d
B) f-d
C) f+d
D) f/d

Answer: Option 3

Explanation:
The distance equation in tacheometry is D=Ks+C.
Multiplying Constant (K): f/i (usually 100).
Additive Constant (C): f+d (usually 0 for anallatic lens).

Q.18 Surveying (Chains)

Question:
What is the length of Engineer's chain?

Options:
A) 100 feet
B) 33 feet
C) 66 feet
D) 50 feet

Answer: Option 1

Explanation:
Engineer's Chain: 100 feet long (100 links).
Gunter's Chain: 66 feet long (100 links).
Revenue Chain: 33 feet long (16 links).
Metric Chain: 20 m or 30 m.

Q.19 Construction Materials (Timber)

Question:
Which group of timber are available in quantity less than 335 m3 per year as per IS: 399-1963?

Options:
A) Group X
B) Group Y
C) Group Z (Implied from context of IS code)
D) (Unclear text)

Answer: Likely Group Z

Explanation:
IS 399 classifies timber based on availability:
Group X: Most common (>1415 					m3/year).
Group Y: Common (355-1415 					m3/year).
Group Z: Less common (< 355 m3/year).
The question asks for less than 335 (likely a typo for 355), which corresponds to Group Z.

Q.20 Surveying (Contours)

Question:
Identify the contour interval (metres) for large scale of map for rolling ground?

Options:
A) 0.5 to 1
B) 1, 2 or 3
C) 0.2 to 0.5
D) 2 to 5

Answer: Option 1

Explanation:
For Large Scale Maps:
Flat Ground: 0.2 m to 0.5 m.
Rolling Ground: 0.5 m to 1 m.
Hilly Ground: 1 m to 2 m.

Q.21 RCC Structures (Columns)

Question:
The IS 456-2000 recommended value of effective length of a RCC compression member of unsupported length L effectively held in position and restrained against rotation in both ends is:

Options:
A) 0.65 L
B) 1.01 L
C) 0.81 L
D) 1.51 L

Answer: Option 1

Explanation:
As per IS 456:2000 (Table 28), for a compression member effectively held in position and restrained against rotation at both ends:
Theoretical effective length: 0.5L
Recommended effective length: 0.65 L

Q.22 Hydraulic Structures (Dams)

Question:
Identify the force in gravity dam that causes stability?

Options:
A) Self-weight of the dam.
B) Water pressure from reservoir.
C) Silt pressure.
D) Uplift pressure.

Answer: Option 1

Explanation:
Self-weight: This is the primary stabilizing force that resists overturning, sliding, and uplift.
Destabilizing forces: Water pressure (horizontal), silt pressure, and uplift pressure all act to destabilize the dam (causing sliding or overturning).

Q.23 Design of Steel Structures (Load Combinations)

Question:
Which load combination satisfies the value of partial safety factor for live loads as 1.05 in case of limit state of strength?

Options:
A) DL + WL/EL
B) DL + ER
C) DL + LL + CL
D) DL + AL + LL

Answer: Option 3

Explanation:
As per IS 800:2007 (Table 4) for Limit State of Strength:
The load combination involving Dead Load (DL) + Imposed Load (LL) + Crane Load (CL) specifies different partial safety factors depending on the probability of simultaneous occurrence.
For the combination of DL + LL + CL, the partial safety factor for the Crane Load (CL) (or accompanying load) is taken as 1.05 in certain service conditions (specifically when considering multiple cranes or specific accompanying probabilities).
Standard combinations like 1.5(DL+LL) or 1.2(DL+LL+WL) do not use 1.05.

Q.24 Environmental Engineering (Sewers)

Question:
What should be the minimum gradient for 250 mm diameter sewer of building drainage when flowing half-full?

Options:
A) 1 in 140
B) 1 in 100
C) 1 in 175
D) 1 in 195

Answer: Option 4

Explanation:
Sewer gradients are designed to maintain a self-cleansing velocity (approx. 0.75 m/s). As per standard tables (e.g., Maguire's rule or CPHEEO manuals) for small diameter sewers:
100 mm dia: 1 in 60
150 mm dia: 1 in 100
200 mm dia: 1 in 175 (approx)
250 mm dia: 1 in 195 to 1 in 250 is the standard recommendation to ensure adequate velocity.

Q.25 Railway Engineering (Rails)

Question:
Which is a merit of bull-headed rails?

Options:
A) B.H. rails are of more strength and stiffness.
B) These rails require light maintenance cost.
C) Removal of tracks is easy with the use of these rails.
D) B.H. rails require no additional cost of iron chairs.

Answer: Option 3

Explanation:
Bull-Headed (B.H.) Rails: The head and foot are of similar size. They sit in chairs fixed to sleepers.
Merit: Because they are held in chairs (not directly spiked), the removal or replacement of the rail is easier and does not disturb the sleeper as much as flat-footed rails.
Demerit: They have less lateral stiffness and require heavy chairs (higher cost).

Q.26 Environmental Engineering (Noise Pollution)

Question:
What is the recommended noise limit for diesel generators for domestic purposes as per environment protection rules?

Options:
A) 75 dB
B) 46 dB
C) 85-90 dB
D) 68 dB

Answer: Option 3

Explanation:
While CPCB norms mandate a maximum of 75 dB(A) at 1 meter for new diesel generator sets (manufacturing standard), the acceptable noise level for domestic generators is often cited in the range of 85-90 dB(A) in various older guidelines or specific contexts (referencing sound power or practical domestic limits). The official answer key for this exam marks 85-90 dB as correct.

Q.27 Building Materials (Bricks)

Question:
When rainwater falls on heated bricks, it distorts the shape of the bricks, and this phenomenon is referred to as:

Options:
A) Blisters
B) Checks
C) Spots
D) Chuffs

Answer: Option 4

Explanation:
Chuffs: The deformation or cracking of hot bricks when rain falls on them is called chuffing. The sudden cooling causes thermal shock and distortion.
Blisters: Caused by trapped air.
Checks: Fine cracks.

Q.28 Building Materials (Cement)

Question:
Which of the following is NOT a characteristic of Portland pozzolana cement (PPC) as compared to ordinary Portland cement (OPC)?

Options:
A) rate of development of early strength within 7 days
B) it reduces the leaching of calcium hydroxide when used in hydraulic structures.
C) offers greater resistance to the attack of aggressive waters
D) produces less heat of hydration

Answer: Option 1

Explanation:
PPC characteristics: Low heat of hydration (True), high resistance to sulphate attack (True), reduces leaching (True), and slower rate of strength gain.
The Odd One Out: Option 1 refers to "rate of development of early strength". PPC has a lower rate of early strength development compared to OPC. If the statement implies "High rate of early strength" (often inferred in such questions), it is not a characteristic of PPC.

Q.29 Construction Technology (Concrete)

Question:
Which of the following is generally provided for transporting concrete from ground level to a lower level?

Options:
A) Cranes
B) Skip and hoist
C) Chute
D) Transit mixer

Answer: Option 3

Explanation:
Chute: An inclined trough or tube used to convey concrete from a higher level (e.g., ground) to a lower level (e.g., a foundation pit) by gravity.
Skip and hoist: Used for lifting concrete up.
Transit mixer: Used for transporting concrete over distances on roads.

Q.30 Building Materials (Lime)

Question:
Select the correct statement related to lime:
Quick lime is obtained by calcination of comparatively pure lime stone.
The lime that sets under water is known as fat lime.

Options:
A) Both statements are correct
B) Statement 1 is correct and 2 is incorrect
C) Both statements are incorrect
D) Statement 1 is incorrect and 2 is correct

Answer: Option 2

Explanation:
Statement 1 (Correct): Quick lime (CaO) is produced by calcining pure limestone (CaCO3).
Statement 2 (Incorrect): The lime that sets under water is known as Hydraulic Lime. Fat lime (high calcium lime) hardens by carbonation with air and does not set under water.

Q.31 Construction Technology (Formwork)

Question:
In normal circumstances and where ordinary Portland cement is used, form work for props to slab spanning up to 4.5 m may generally be removed after the expiry of:

Options:
A) 24 hours to 48 hours
B) 28 days
C) 7 days
D) 14 days

Answer: Option 3 (7 days)

Explanation:
As per IS 456:2000 (Clause 11.3.1), the stripping time for formwork using OPC is:
Vertical formwork to columns, walls, beams: 16-24 hours.
Props to slabs (spanning up to 4.5 m): 7 days.
Props to slabs (spanning over 4.5 m): 14 days.
Props to beams and arches (spanning up to 6 m): 14 days.

Q.32 Concrete Technology (Testing)

Question:
What is the minimum pre-hardening period at 20°C for M25 Grade of concrete made with OPC?

Options:
A) 22 hours
B) 24 hours
C) 20 hours
D) 17 hours

Answer: Option 2 (24 hours)

Explanation:
For standard testing of concrete strength, specimens are typically demolded after 24 hours (± 30 minutes) and then cured. This initial period allows the concrete to gain sufficient strength (harden) to withstand handling without damage.

Q.33 Concrete Technology (Cement Types)

Question:
Which grade of OPC is most popular cement and is widely used for structural works, RCC works, and non-structural works such as plastering, flooring etc?

Options:
A) Grade 53
B) Grade 43
C) Grade 23
D) Grade 33

Answer: Option 2 (Grade 43)

Explanation:
OPC 43 Grade (IS 8112): This is the most commonly used general-purpose cement for RCC work, plastering, and masonry. It offers a balance of strength and heat of hydration.
OPC 33 Grade: Now rarely used for structural works due to lower strength.
OPC 53 Grade: Used for high-strength concrete and pre-stressed concrete but requires stricter curing due to faster hydration.

Q.34 Surveying (Scale Errors)

Question:
On a plan that was scaled at 1 cm = 40 metres, a surveyor measured the distance between two spots, and the result was 450 metres. But afterwards he realised he had been using a scale of 1 cm = 20 m. Then actual distance between the spots is:

Options:
A) 925 m
B) 825 m
C) 900 m
D) 875 m

Answer: Option 3 (900 m)

Explanation:
Measurement: The surveyor used a scale of 1 cm = 20 m to read a value of 450 m.
Map Distance = 45020=22.5 cm
Actual Calculation: The map's actual scale is 1 cm = 40 m.
Actual Distance = 22.5 cm×40 m/cm=900 m

Q.35 Environmental Engineering (Water Treatment)

Question:
Temporary hardness of water can be removed by:

Options:
A) Zeolite process
B) Boiling process
C) Deionization process
D) Lime-soda process

Answer: Option 2 (Boiling process)

Explanation:
Temporary Hardness: Caused by bicarbonates of calcium and magnesium. It can be removed simply by boiling, which converts bicarbonates into insoluble carbonates (Ca(HCO3)2→CaCO3↓+H2O+CO2).
Permanent Hardness: Removed by Zeolite, Lime-soda, or Ion exchange methods.

Q.36 Soil Mechanics (Index Properties)

Question:
Identify one of the cohesive index property of soils?

Options:
A) In-place density
B) Shape of particles
C) Clay content
D) Atterberg limits

Answer: Option 4 (Atterberg limits)

Explanation:
Atterberg limits (Liquid Limit, Plastic Limit, Shrinkage Limit) are used to characterize the consistency and plasticity of fine-grained (cohesive) soils. They indicate the range of water content over which the soil behaves plastically. "Shape of particles" is more relevant to coarse-grained soils.

Q.37 Estimating and Costing

Question:
Which of the following is NOT an approximate method of estimate?

Options:
A) Quantity estimate
B) Plinth area estimate
C) Unit rate estimate
D) Cubic rate estimate

Answer: Option 1 (Quantity estimate)

Explanation:
Approximate Estimates: Plinth area, Cubic content, and Unit base methods are used for preliminary cost assessments.
Quantity Estimate (Detailed Estimate): This involves working out the exact quantities of each item of work from drawings and multiplying by unit rates. It is an accurate method, not approximate.

Q.38 Fluid Mechanics (Pumps)

Question:
What is the advantage of reciprocating pump over centrifugal pump?

Options:
A) Require small space for installation.
B) Initial cost is much more in this pump.
C) Gives high pressure at the outlet.
D) Low wear and tear, so low maintenance.

Answer: Option 3 (Gives high pressure at the outlet)

Explanation:
Reciprocating Pumps (Positive Displacement): They are capable of developing very high pressures (heads) even at low flow rates.
Centrifugal Pumps: Better for high discharge at low to medium heads. They typically require less space and have lower maintenance than reciprocating pumps.

Q.39 Concrete Technology (Hot Weather Concreting)

Question:
What is the threshold temperature to be considered for hot weather concreting at the time of placing as per IS 7861-part-1-1975?

Options:
A) Greater than 30∘C
B) Greater than 60∘C
C) Greater than 50∘C
D) Greater than 40∘C

Answer: Option 4 (Greater than 40∘C)

Explanation:
IS 7861 (Part 1): 1975 deals with hot weather concreting. It specifies that the temperature of concrete at the time of placing should generally not exceed 40°C. High temperatures accelerate setting, increase water demand, and can lead to thermal cracking.

Q.40 Fluid Mechanics (Properties)

Question:
Which is the physical property of fluid also termed as mass density?

Options:
A) Fluid density
B) Specific weight
C) Specific volume
D) Weight density

Answer: Option 1 (Fluid density)

Explanation:
Mass Density (ρ): Mass per unit volume (kg/m³). Often simply called Density.
Specific Weight (γ): Weight per unit volume (N/m³). Also called Weight Density.
Specific Volume (v): Volume per unit mass (m³/kg). Reciprocal of density.

Q.41 Water Resources (Reservoir Sedimentation)

Question:
Which of the following statement for silting process in reservoirs is true?

Options:
A) It is initiated when the river damming is partially completed.
B) This silting process is reversible process.
C) It is very simple process of entrainment and transport of sediment.
D) The silting process is naturally caused and anthropically influenced.

Answer: Option 4

Explanation:
Silting (sedimentation) is a natural geological process where soil and rock particles are transported by water and settle in the reservoir. However, it is significantly anthropically influenced (influenced by human activity) because activities like deforestation, agriculture, and construction in the catchment area increase soil erosion and the sediment load entering the river.

Q.42 Highway Materials (Bitumen)

Question:
Which type of cutback bitumen is recommended for premix with less quantity of fine aggregates?

Options:
A) Rapid curing
B) Medium curing
C) Very slow curing
D) Slow curing

Answer: Option 2

Explanation:
Medium Curing (MC) cutbacks contain a volatile diluent (like kerosene) and are recommended for premix with appreciable quantity of fines or for open-graded mixes where a longer setting time is needed to allow for mixing and placing compared to Rapid Curing.

Rapid Curing (RC) is used for surface dressing/patchwork.
Slow Curing (SC) is used for fine-graded mixes or soil stabilization.

Q.43 Environmental Engineering (Air Pollution)

Question:
Which air pollutants are generally produced by nuclear explosions, war explosive etc?

Options:
A) Nitrogen-oxide
B) Radioactive pollutants
C) Volatile organic compounds
D) Chlorofluorocarbons

Answer: Option 2

Explanation:
Nuclear explosions and accidents release radioactive pollutants (e.g., Uranium-235, Strontium-90, Iodine-131, Cesium-137) into the atmosphere. These are distinct from conventional pollutants like NOx or CFCs.

Q.44 Irrigation Engineering (Drainage)

Question:
In which system of tile drainage, the field drains are parallel to each other on each side of the lateral?

Options:
A) Intercepting system
B) Double main system
C) Herringbone system
D) Grid iron system

Answer: Option 4

Explanation:
Grid Iron System: Consists of a main drain with parallel field drains (laterals) entering it from one side, usually at right angles. It is suitable for flat land.

The Herringbone system has laterals entering the main from both sides at an angle.

Q.45 Surveying (Chains)

Question:
The variety of 66 ft. long chain where each link being 0.6 ft. long is termed as:
(Note: 0.6 ft is likely a typo for 0.66 ft in the question text)

Options:
A) Metric chains
B) Revenue chains
C) Gunter's chain
D) Engineer's chain

Answer: Option 3

Explanation:
Gunter's Chain is 66 feet long and consists of 100 links.
Length of one link = 66/100=0.66 ft (approx 7.92 inches).

Engineer's chain is 100 ft long.
Revenue chain is 33 ft long.

Q.46 Railway Engineering (Track)

Question:
If the sleeper density is M+7 on a broad gauge route and the length of the rail is 13 m, then how many sleepers are required for 1.3 kilometer length?

Options:
A) 2150
B) 1850
C) 2000
D) 1700

Answer: Option 3

Explanation:
Sleeper Density: M+7.
Rail Length (M): 13 m.

Sleepers per rail: 13+7=20 sleepers

Total Track Length: 1.3 km = 1300 m

Number of Rails: 1300 m/13 m=100 rails.

Total Sleepers: 100 rails×20 sleepers/rail=2000 sleepers

Q.47 Fluid Mechanics (Properties)

Question:
What is S.I. unit for kinematic viscosity of fluid?

Options:
A) Newton.meter/square second (N.m/s2)
B) Square meter/second (m2/s)
C) Newton/second (N/s)
D) Newton.second/square meter (N.s/m2)

Answer: Option 2

Explanation:
Kinematic Viscosity (ν): Defined as dynamic viscosity divided by density (μ/ρ).

SI Unit: Square meter per second (m2/s).

Option 4 (N.s/m2 or Pa.s) is the unit for Dynamic Viscosity.

Q.48 Geotechnical Engineering (Bearing Capacity)

Question:
If a strip footing 1.5 m wide and situated at a depth of 1 m in clay has a gross bearing capacity of 350 kN/m², its net bearing capacity for gamma 30 kN/m² equals.

Options:
A) 330 					kN/m2
B) 320 					kN/m2
C) 300 					kN/m2
D) 310 					kN/m2

Answer: Option 2

Explanation:
The relationship between Gross (qg) and Net (qn) bearing capacity is:

qn=qg-γDf

Given: qg=350 					kN/m2.

Soil Unit Weight (γ): 30 					kN/m3 (Note: The unit in question says kN/m2, likely a typo for kN/m3).

Depth (Df): 1 m

Calculation: 350-(30×1)=320 					kN/m2

Q.49 Fluid Mechanics (Fluid Statics)

Question:
Which of the following rule is the application concept in hydraulic press?

Options:
A) Pascal's law
B) Archimedes' principle
C) Bernoulli's principle
D) Simon's rule

Answer: Option 1

Explanation:
Pascal's Law states that a change in pressure applied to an enclosed fluid is transmitted undiminished to all portions of the fluid and to the walls of its container. This principle allows a hydraulic press to multiply force.

Q.50 Soil Mechanics (Compaction)

Question:
Which is the factor that affect the compaction of soil, that when increased, the electrical double layer expands and the interparticle repulsive forces increase, resulting in higher dry density?

Options:
A) Water content
B) Amount of compaction
C) Method of compaction
D) Type of soil

Answer: Option 1

Explanation:
Water Content:
Adding water lubricates the soil particles.
In fine-grained soils (clays), increasing water content towards the Optimum Moisture Content (OMC) affects the electrical double layer. On the wet side of optimum, the double layer expands, causing increased repulsion between particles. This leads to a dispersed structure (parallel orientation) which allows particles to slide over each other and pack into a denser configuration (higher dry density) under the compaction effort.

Q.51 Structural Analysis (Fixed Beams)

Question:
In case of fixed beam, the area of B.M. diagram due to end moments is:

Options:
A) less than the area of B.M. diagram due to vertical loads.
B) equal to the area of B.M. diagram due to horizontal loads.
C) more than the area of B.M. diagram due to vertical loads.
D) equal to the area of B.M. diagram due to vertical loads.

Answer: Option 4

Explanation:
For a fixed beam subjected to vertical loading, the net change in slope between the two fixed ends is zero (Mohr's First Theorem). This implies that the area of the free bending moment diagram (due to vertical loads acting on a simply supported span) must be equal to the area of the fixed bending moment diagram (due to end moments).

Areafree=Areafixed

Q.52 Engineering Geology (Rocks)

Question:
Which type of rocks possess coarsely crystalline structure?

Options:
A) Hypabyssal igneous rocks
B) Volcanic igneous rocks
C) Plutonic igneous rocks
D) Aqueous igneous rocks

Answer: Option 3

Explanation:
Plutonic Rocks (Intrusive): These cool slowly deep beneath the earth's surface (e.g., Granite). The slow cooling allows large crystals to form, resulting in a coarse-grained (crystalline) structure.
Volcanic Rocks (Extrusive): Cool quickly on the surface (e.g., Basalt), resulting in fine-grained structures.

Q.53 Highway Engineering (Vertical Curves)

Question:
The summit curves are required to be introduced at the situations where:

Options:
A) a negative grade meets a level stretch.
B) a positive grade meets another positive grade.
C) a negative grade meets a steeper positive grade.
D) a positive grade meets another milder positive grade.

Answer: Option 4

Explanation:
A summit curve (convex curve) is required when there is an upward change in gradient or when the gradient decreases.
Option 4: An upward slope (+n1) meets a less steep upward slope (+n2, where n1>n2). This forms a convex hump, requiring a summit curve.
Option 1 & 3 form valley curves (sag curves).

Q.54 Concrete Technology (Grades)

Question:
Which group of concrete has characteristic strength of 35 N/mm² for 150 mm cube characteristic compressive strength at 28 days?

Options:
A) High strength concrete
B) Standard concrete
C) Ordinary concrete
D) Very high strength concrete

Answer: Option 2

Explanation:
As per IS 456:2000:
Ordinary Concrete: M10 – M20
Standard Concrete: M25 – M55 (includes M35)
High Strength Concrete: M60 – M80 (and above in newer revisions).

Q.55 Structural Analysis (Bending Moment)

Question:
A continuous beam ABC 10 m long rests on three supports A, B, and C at the same level and is loaded as shown in figure. Find the bending moment at point D?

Options:
A) 1.4 kN-m
B) 6 kN-m
C) 2 kN-m
D) 8 kN-m

Answer: Option 2

Explanation:
The official answer provided is 6 kN-m. In continuous beams, Bending Moments are typically calculated using the Theorem of Three Moments (Clapeyron's Theorem) or moment distribution methods to find support moments first, then free moments at specific points like D.

Q.56 Surveying (Theodolite)

Question:
The line of sight of a transit must be perpendicular to:

Options:
A) The horizontal axis at its intersection with the vertical axis.
B) The axis of altitude level at its intersection with striding level.
C) The axis of striding level at its intersection with plate level.
D) The vertical axis at its intersection with altitude level.

Answer: Option 1

Explanation:
For a theodolite (transit) to be in proper adjustment:
Line of Sight (Collimation Axis) must be perpendicular to the Horizontal Axis (Trunnion Axis).
Horizontal Axis must be perpendicular to the Vertical Axis.

Q.57 Railway Engineering (Fastenings)

Question:
Rail-to-rail fastenings involve the use of ____ to hold two rails together in both the horizontal and vertical planes.

Options:
A) Fang bolts
B) Bearing plates
C) Dog spike
D) Fish plates

Answer: Option 4

Explanation:
Fish plates are used to join two rail ends together (rail-to-rail fastening) to maintain continuity. They are bolted to the web of the rails to hold them in position both horizontally and vertically.

Q.58 Irrigation Engineering (Methods)

Question:
Which of the following irrigation techniques, the evaporation and loss of water due to the wind is negligibly low?

Options:
A) Trickle irrigation
B) Sprinkler irrigation
C) Flood irrigation
D) Furrow irrigation

Answer: Option 1

Explanation:
Trickle (Drip) Irrigation applies water directly to the root zone of the plants through emitters. Since water is not sprayed into the air (like sprinkler) and does not stand in large open areas (like flood), evaporation and wind drift losses are minimized.

Q.59 Structural Analysis (Theorems)

Question:
Clapeyron theorem is used to analyze which type of beams?

Options:
A) Fixed beam
B) Continuous beam
C) Cantilever beam
D) Simply supported beam

Answer: Option 2

Explanation:
Clapeyron's Theorem of Three Moments is a classical method used specifically for the analysis of continuous beams (beams with more than two supports) to find the moments at the supports.

Q.60 Environmental Engineering (Sewerage)

Question:
According to IS: 4111 (Part 1)-1986, what is the spacing of manholes that may be allowed on straight runs for sewers of diameter above 1.5 m to 2.0 m?

Options:
A) 50 to 90 m
B) 150 to 200 m
C) 90 to 150 m
D) 200 to 300 m

Answer: Option 4

Explanation:
As per IS 4111 (Part 1), spacing of manholes depends on sewer diameter:
Small sewers: 30 m.
0.9 m to 1.5 m dia: 90 m to 150 m.
Above 1.5 m to 2.0 m dia: Spacing can be 200 m to 300 m (since these are large enough for man-entry for cleaning).

Q.61 Building Materials (Bricks)

Question:
A field test for strength is that a brick shall not break into pieces on dropping the brick on a hard ground from:

Options:
A) 0.9–1.0 m
B) 0.4–0.6 m
C) 1.1–1.5 m
D) 0.6–0.8 m

Answer: Option 1

Explanation:
A standard field test for the toughness and strength of a good brick is that it should not break when dropped flat on hard ground from a height of approximately 1 meter (Standard practice says ~3 to 4 feet, i.e., 0.9–1.0 m).

Q.62 Construction Materials (Properties)

Question:
Which of the following describes the crucial mechanical properties of construction material?

Options:
A) Refractoriness
B) Durability
C) Bending
D) Hygroscopicity

Answer: Option 3

Explanation:
Bending (Flexural Strength): This is a mechanical property indicating the material's ability to resist deformation under load.
Refractoriness is a thermal property.
Hygroscopicity is a physical/chemical property (interaction with water).
Durability is a general serviceability characteristic.

Q.63 Irrigation Engineering (Water Requirement)

Question:
If wheat requires about 7.5 cm of water after every 28 days, and the base period for wheat is 140 days, find out the value of delta for wheat.

Options:
A) 34 cm
B) 30 cm
C) 37.5 cm
D) 26.5 cm

Answer: Option 3

Explanation:
Water per watering: 7.5 cm.
Frequency: Every 28 days.
Base Period: 140 days.
Number of waterings: 140/28=5.
Total Delta (Δ): 5×7.5 cm=37.5 cm.

Q.64 Soil Mechanics (Permeability)

Question:
A constant-head permeameter is used for:

Options:
A) Clayey soils
B) Coarse-grained soils
C) Organic soils
D) Silty soils

Answer: Option 2

Explanation:
Constant Head Test: Used for coarse-grained soils (Gravel, Sand) where permeability is high (k>10-4 cm/sec).
Falling Head Test: Used for fine-grained soils (Silt, Clay) where permeability is low.

Q.65 Environmental Engineering (Wastewater Treatment)

Question:
Which wastewater treatment operation helps in oxidation of organic matter to reduce BOD?

Options:
A) Biological processes
B) Mechanical separation
C) Coagulation
D) Boiling

Answer: Option 1

Explanation:
Biological processes (like Activated Sludge Process, Trickling Filters) utilize microorganisms to oxidize (consume) biodegradable organic matter, thereby reducing the Biochemical Oxygen Demand (BOD) of the wastewater.

Q.66 Concrete Technology (Cement Testing)

Question:
What should be the minimum specific surface of OPC?

Options:
A) $3250~cm^2/g$
B) $3000~cm^2/g$
C) $4000~cm^2/g$
D) $2250~cm^2/g$

Answer: Option 4

Explanation:
As per IS 8112 (43 Grade OPC) and IS 269 (33 Grade OPC), the specific surface area (fineness) measured by the Blaine's Air Permeability method should not be less than 225 m²/kg (which equals 2250 cm²/g).

Q.67 Steel Structures (Shear)

Question:
According IS 800-2007, the nominal plastic shear resistance under pure shear is given by: (Av = shear area, fyw = yield strength of web).

Options:
A) Vn=Avfyw3
B) Vn=Avfyw5
C) Vn=Avfyw2
D) Vn=Avfyw7

Answer: Option 1

Explanation:
As per IS 800:2007 (Clause 8.4), the design shear strength is based on the Von Mises yield criterion, which assumes yield in shear occurs at fy/3.
Formula: Vn=Avfyw/3.

Q.68 Concrete Technology (Cement Types)

Question:
Match the following related to type of cement with application:

Options:
A) 1-C; 2-D; 3-A; 4-B
B) 1-C; 2-D; 3-B; 4-A
C) 1-D; 2-C; 3-A; 4-B
D) 1-C; 2-A; 3-D; 4-B

Answer: Option 1

Explanation:
Rapid Hardening: C. Cold weather concrete (generates heat faster).
Sulphate Resisting: D. Marine conditions (resists sulphate attack from sea water).
Refractory: A. Electric furnaces (withstands high temps).
Low Heat: B. Massive dams (prevents thermal cracking).

Q.69 Water Supply Engineering (Demand)

Question:
In water supply engineering, the maximum hourly demand of water is equal to:

Options:
A) 1.48 x (Annual average weekly demand)
B) 2.7 x (Annual average hourly demand)
C) 1.8 x (Annual average daily demand)
D) 1.28 x (Annual average monthly demand)

Answer: Option 2

Explanation:
Max Daily Demand = 1.8 × Avg. Daily Demand.
Max Hourly Demand = 1.5 × Max Daily Demand.
Therefore, Max Hourly = 1.5×(1.8×Avg. Daily)=2.7×Avg. Daily Demand (when expressed as an hourly rate, i.e., Avg. Annual Hourly Demand).

Q.70 Highway Engineering (Classification)

Question:
Which among the following is a road type categorized as secondary system of roads?

Options:
A) Village roads
B) Other district roads
C) State highways
D) National highways

Answer: Option 3

Explanation:
According to the Nagpur Road Plan classification:
Primary System: National Highways (NH), Expressways.
Secondary System: State Highways (SH) and Major District Roads (MDR).
Tertiary System: Other District Roads (ODR) and Village Roads (VR).

Q.71 Highway Engineering (Bitumen Testing)

Question:
In which of the following bitumen adhesion test, the stripping value of aggregates should not exceed 25 percent when aggregate coated with bitumen is immersed in water bath at 40°C for 24 hours?

Options:
A) Dynamic immersion test
B) Immersion trafficking test
C) Immersion mechanical test
D) Static immersion test

Answer: Option 4

Explanation:
The Static Immersion Test involves coating aggregates with bitumen and immersing them in water at 40°C for 24 hours. The stripping value (percentage of uncovered area) should generally not exceed 25% (or 5% for superior grades depending on specifications, but the question describes the static test procedure).

Q.72 Fluid Mechanics (Turbines)

Question:
In the case of Pelton turbine installed in a hydraulic power plant, the gross head available is the vertical distance between:

Options:
A) Reservoir level and tail race
B) Reservoir level and turbine inlet
C) Forebay and the tail race
D) Forebay and turbine inlet

Answer: Option 2

Explanation:
Gross Head (Hg): It is the difference between the head race level (reservoir level) and the water level at the turbine inlet (or nozzle center for Pelton wheel).
Net Head: Gross head minus hydraulic losses (friction in penstock).

Q.73 Highway Engineering (Right of Way)

Question:
What will be the normal right-of-way width of rural areas with plain and rolling terrain for village roads?

Options:
A) 12 m
B) 8 m
C) 15 m
D) 25 m

Answer: Option 1

Explanation:
As per IRC (Indian Roads Congress) recommendations for Village Roads in plain and rolling terrain:
Normal Right-of-Way: 12 m
Range: 12 m to 18 m.

Q.74 Building Materials (Bricks)

Question:
Consider the following operations of preparation of brick earth:

- Digging

- Weathering

- Tempering

- Blending

- Unsoiling
The correct sequence of these operations are:

Options:
A) 5, 1, 4, 2, 3
B) 5, 1, 2, 4, 3
C) 1, 5, 2, 4, 3
D) 5, 1, 3, 2, 4

Answer: Option 2

Explanation:
The standard sequence for preparing brick earth is:
Unsoiling: Removing top vegetation/soil.
Digging: Excavating the earth.
Weathering: Exposing soil to atmosphere for softening.
Blending: Mixing different soils/ingredients.
Tempering: Adding water and kneading (pug mill).
Order: 5 → 1 → 2 → 4 → 3.

Q.75 Building Materials (Bricks)

Question:
Identify the minimum compressive strength in MPa for third class bricks?

Options:
A) 10
B) 8
C) 5.0
D) 3.5

Answer: Option 4

Explanation:
First Class Bricks: Minimum 10.5 N/mm² (MPa).
Second Class Bricks: Minimum 7.0 N/mm² (MPa).
Third Class (Common) Bricks: Minimum 3.5 N/mm² (MPa).

Q.76 RCC Design (Bond Stress)

Question:
What should be the Design Bond Stress in Limit State Method for plain bars in tension for M35 grade of concrete?

Options:
A) 1.4
B) 1.7
C) 1.5
D) 1.2

Answer: Option 2

Explanation:
As per IS 456:2000 (Clause 26.2.1.1), Design bond stress (τbd) for plain bars in tension:
M20: 1.2 MPa
M25: 1.4 MPa
M30: 1.5 MPa
M35: 1.7 MPa
M40 and above: 1.9 MPa

Q.77 Engineering Geology (Rocks)

Question:
Identify an example of ultra-basic igneous rocks?

Options:
A) Peridotite
B) Syenite
C) Gabbro
D) Rhyolite

Answer: Option 1

Explanation:
Ultra-basic rocks: Silica content < 45% (e.g., Peridotite, Dunite).
Basic rocks: Silica 45-55% (e.g., Basalt, Gabbro).
Intermediate rocks: Silica 55-65% (e.g., Syenite).
Acidic rocks: Silica > 65% (e.g., Granite, Rhyolite).

Q.78 Soil Mechanics (Classification)

Question:
Identify the diameter of one of the soils separate i.e., gravel?

Options:
A) 4.75–80 mm
B) 80–300 mm
C) 0.002 mm
D) 0.002–0.075 mm

Answer: Option 1

Explanation:
As per IS Soil Classification:
Boulders: > 300 mm
Cobbles: 80 mm – 300 mm
Gravel: 4.75 mm – 80 mm
Sand: 0.075 mm – 4.75 mm
Silt/Clay: < 0.075 mm

Q.79 Fluid Mechanics (Pressure)

Question:
The pressure difference of two very light gases in two rigid vessels is being measured by a vertical U-tube water filled manometer. The reading is found to be 10 cm. What is the pressure difference?

Options:
A) 0.981 bar
B) 981 N/m²
C) 98.1 Pa
D) 9.81 kPa

Answer: Option 2

Explanation:
Formula: ΔP=ρgh
ρ (water) = 1000 kg/m3
g=9.81 m/s2
h=10 cm=0.1 m
Calculation: ΔP=1000×9.81×0.1=981 N/m2 (Pa)

Q.80 Environmental Engineering (Wastewater)

Question:
The Activated Sludge Process is to remove ____ present in the municipal or industrial waste water.

Options:
A) Dissolved oxygen
B) Total dissolved solid
C) Biochemical oxygen demand
D) Chemical oxygen demand

Answer: Option 3

Explanation:
The primary goal of the Activated Sludge Process (a biological treatment) is to reduce the Biochemical Oxygen Demand (BOD) and suspended solids by oxidizing biodegradable organic matter.

Q.1 Syllogism

Question:
Statements: (A) All paints are black. (B) No paint is colour.
Conclusions: I. No colour is black. II. Some colours are black.

Options:
A) Neither conclusion I nor II follows
B) Only conclusion I follows
C) Only conclusion II follows
D) Either conclusion I or II follows

Answer: Option 4

Explanation:
From statements: Paints are a subset of Black. Paints and Colour are disjoint (no overlap).
Relationship between Colour and Black: We know Paints are inside Black, and no Paint is Colour. However, Colour could be completely outside Black, or it could overlap with the part of Black that isn't Paint.
Conclusion I (No colour is black) and Conclusion II (Some colours are black) form a complementary pair (E and I type propositions). Since the relationship is undetermined, one of them must be true. Hence, Either I or II follows.

Q.2 Non-Verbal (Figure Series)

Question:
Select the figure from among the given options that can replace the question mark (?) in the following series.

Answer: Option 2

(Note: Visual series requires observing the rotation/movement of elements in the provided images.)

Q.3 Analogy

Question:
USA : Congress :: India : ?

Options:
A) Parliament
B) President
C) Constitution
D) Prime Minister

Answer: Option 1

Explanation:
The legislative body of the USA is called Congress. Similarly, the legislative body of India is called Parliament (Sansad).

Q.4 Number Series

Question:
36, 75, 147, 297, 591, ?

Options:
A) 1273
B) 1185
C) 1179
D) 1279

Answer: Option 2

Explanation:
Pattern:
36×2+3=75
75×2-3=147
147×2+3=297
297×2-3=591
Next: 591×2+3=1182+3=1185

Q.5 Seating Arrangement

Question:
Eight persons (A-H) sitting in a straight line facing north. C is 3rd to left of D. B is 2nd to left of C. E is immediate left of A. D is 2nd to left of F. G is immediate right of D. Who is immediate right of H?

- D

- C

- A

- G

Answer: Option 2

(Note: The exact option text is not fully visible, but the logic requires solving the linear arrangement.)

Q.6 Letter Logic

Question:
(RST, MML, HGD) -> Logic?

Options:
A) PUT, KOL, FHD
B) PQN, KKF, FEY
C) MNK, GFC, BZU
D) TUN, OOF, JIX

Answer: Option 4

Explanation:
Check the gap between letters in groups. Official Answer is Option 4.

Q.7 Coding Decoding

Question:
"REPLAY" is coded as "BZOKVI" and "NOVELS" is coded as "HOVELM". How will "DEPUTY" be coded?

Options:
A) WVFKGB
B) BGFKVW
C) KVWBGF
D) WVKFGB

Answer: Option 2

Explanation:
Likely a Reverse/Opposite Letter logic or a specific shift pattern.
Opposites: D→W, E→V, P→K, U→F, T→G, Y→B.
Reverse them: B G F K V W.

Q.8 Letter Cluster Series

Question:
CBDA, GFHE, KJLI, ONPM, SRTQ, ?

Options:
A) WVXU
B) YVXW
C) XUWV
D) XWYV

Answer: Option 1

Explanation:
Add 4 to the letters in each position. First letters: C→G→K→O→S→W. Second letters: B→F→J→N→R→V, etc.

Q.9 Non-Verbal (Figure Matrix)

Question:
Select the figure which can be placed at the sign of question mark (?)

Answer: Option 1

Q.10 Number Series

Question:
4, 15, 2, 17, ?, 19

Options:
A) 0
B) -1
C) 3
D) 5

Answer: Option 1

Explanation:
This is an alternating series.
Series 1 (1st, 3rd, 5th terms): 4, 2, ? → Decreasing by 2. Next is 0.
Series 2 (2nd, 4th, 6th terms): 15, 17, 19 → Increasing by 2.

Q.1 Profit and Loss

Question:
A ration shop dealer sells his sugar using weights that are 10% less than the true weight and makes a profit of 15%. Find his total gain percentage.

Options:
A) 27.8%
B) 29.8%
C) 30.2%
D) 28.8%

Answer: Option 1

Explanation:
Let the Cost Price (CP) of 1000g sugar be ₹1000.
Dealer charges: He marks up by 15%, so Selling Price (SP) for "1kg" = ₹1150.
Actual Goods given: He uses 10% less weight, so he gives 900g.
Cost to Dealer: CP of 900g = ₹900.
Gain: ₹1150 - ₹900 = ₹250.
Gain %: 250900×100=27.77%≈27.8%.

Q.2 Discounts

Question:
Find the single discount to a series 25%, 15% and 10% (Correct to two decimals).

Options:
A) 41.25%
B) 42.63%
C) 40.12%
D) 40.35%

Answer: Option 2

Explanation:
Let Marked Price = 100.
After 25% discount: 100×0.75=75
After 15% discount: 75×0.85=63.75
After 10% discount: 63.75×0.90=57.375
Total Discount: 100-57.375=42.625%
Rounding to two decimals: 42.63%

Q.3 Simple Interest

Question:
The rates of simple interest in two Nationalized banks are in the ratio 3:2. Rani wants to deposit her total savings in two banks in such a way that she receives equal quarterly interest from both. She should deposit the savings in bank A and B in the ratio:

Options:
A) 2:1
B) 3:2
C) 1:2
D) 2:3

Answer: Option 4

Explanation:
Let amounts be P1 and P2, and rates be R1=3x and R2=2x.
Interest is equal (SI1=SI2):
P1×R1=P2×R2 (Time is constant)
P1×3=P2×2
P1/P2=2/3
Ratio is 2:3.

Q.4 Time, Speed and Distance

Question:
In a morning walk Rama walks at 12 kmph instead of kmph, he would have walked 16 km more. The actual distance travelled by him is in km.
(Note: The original speed value is obscured in the text but calculated as 8 kmph based on the answer).

Options:
A) 32
B) 30
C) 33
D) 31

Answer: Option 1

Explanation:
Let actual speed be S and time be t.
Actual Distance D=S×t.
New Speed = 12 kmph. New Distance = D+16.
Time is constant.
DS=D+1612
Using the answer (Actual Distance = 32 km):
If D=32, then 32S=32+1612=4 hours.
So actual speed S=32/4=8 kmph.
Since 32 km fits the logic for a whole number speed, the answer is 32 km.

Q.5 Profit and Loss

Question:
Vinay bought 100 pens (all at same cost price). He sold 40 pens at a profit of 20 percent, 20 pens at a profit of 40 percent and the remaining at the cost price. If he had sold all at a profit of 20 percent, then he would have got Rs. 160 more. What is the cost price of each pen?

Options:
A) Rs. 45
B) Rs. 36
C) Rs. 30
D) Rs. 40

Answer: Option 4

Explanation:
Let CP of 1 pen = x. Total CP = 100x.
Case 1 (Actual Sale):
Profit on 40 pens: 40x×0.20=8x
Profit on 20 pens: 20x×0.40=8x
Profit on remaining 40: 0
Total Profit = 16x
Case 2 (Hypothetical):
Profit on 100 pens at 20%: 100x×0.20=20x
Difference:
20x-16x=160
4x=160⇒x=40
Cost Price per pen = Rs. 40.

Q.6 LCM and HCF

Question:
The sum of two numbers is 1580 and their LCM is 18816. Find the two numbers.

Options:
A) 9, 789
B) 13, 562
C) 12, 1568
D) 11, 1245

Answer: Option 3

Explanation:
Check Sum: 12+1568=1580. (Matches).
LCM = 12×1568/GCD(12,1568)=1568 (GCD is 4, so LCM = 12 x 1568 / 4 = 4704). The question seems to have inconsistency, but Option 3 is the only pair that sums to 1580.

Q.7 Mensuration

Question:
A cuboid whose sides are 8 cm, 9 cm and 24 cm is melted to form a new cube. What is the respective ratio of the total surface area of the cuboid and cube?

Options:
A) 10:9
B) 13:8
C) 17:12
D) 7:5

Answer: Option 1

Explanation:
Volume: 8×9×24=1728 cm3.
Side of Cube (a): a3=1728⇒a=12 cm.
TSA of Cuboid: 2(lb+bh+hl)=2(72+216+192)=2(480)=960 cm2.
TSA of Cube: 6a2=6×144=864 cm2.
Ratio: 960:864. Dividing both by 96 gives 10 : 9.

Q.8 Fractions

Question:
Reduce to lowest form: 295565

Options:
A) 59113
B) 39123
C) 59123
D) 49113

Answer: Option 1

Explanation:
Divide numerator and denominator by 5:
295÷5=59
565÷5=113
Result: 59/113

Q.9 Percentages

Question:
If a number X is 56 percent less than another number Y and Y is 10 percent more than 3000, then what is the value of X?

Options:
A) 1476
B) 1452
C) 1398
D) 1502

Answer: Option 2

Explanation:
Y=3000+10% of 3000=3300.
X is 56% less than Y, meaning X is 44% of Y.
X=0.44×3300=1452.

Q.10 Ratio and Proportion

Question:
Rs. 169000 is divided among A, B and C such that A's share is 6/7 of total share of B and C together and B's share is 4/9 of total share of A and C together. What is the share of C?

Options:
A) Rs. 31000
B) Rs. 11000
C) Rs. 39000
D) Rs. 21000

Answer: Option 3

Explanation:
A=67(B+C)⇒AA+B+C=613.
A=613×169000=78,000.
B=413×169000=52,000.
C=169000-(78000+52000)=39,000.

Q.1 History

Question:
In which year did the Lahore Congress adopt the demand for 'Purna Swaraj'?

Options:
A) December 1931
B) December 1929
C) December 1928
D) December 1930

Answer: Option 2

Explanation:
The Indian National Congress passed the historic 'Purna Swaraj' (Total Independence) resolution at its Lahore session in December 1929, presided over by Jawaharlal Nehru.

Q.2 Art & Culture

Question:
Who among the following learned about the Dhrupad style of composing and Hindustani classical music in Benares when he was a student of Chidambaranatha Yogi?

Options:
A) Syama Sastry
B) Saint Thyagaraja
C) Muthuswami Dikshitar
D) Sonti Venkataramana

Answer: Option 3

Explanation:
Muthuswami Dikshitar, one of the Trinity of Carnatic music, lived in Benares for several years with his guru Chidambaranatha Yogi, where he was exposed to Hindustani music, particularly the Dhrupad style.

Q.3 History

Question:
Which of the following is CORRECTLY matched? (Emperor - Tenure)

Options:
A)

B)

C)

D)

Answer: Option 2

Explanation:
Ghiyasuddin Tughluq: 1320–1325
Muhammad bin Tughluq: 1325–1351
Firuz Shah Tughluq: 1351–1388

Q.4 Science & Technology

Question:
[CSIR-NAL] has developed a medium-class BVLOS (Beyond Visual Line of Sight) multi-copter UAV.

Options:
A) CSIR-CRRI
B) CSIR-CMERI
C) CSIR-NAL
D) CSIR-IHBT

Answer: Option 3

Explanation:
CSIR-National Aerospace Laboratories (NAL) developed the multi-copter drone capable of BVLOS operations, often used for medical transport and aerial monitoring.

Q.5 Economy

Question:
The Gross Value Added is estimated from GDP by adding subsidies on production and subtracting ______.

Options:
A) depreciation
B) net export
C) indirect taxes
D) net import

Answer: Option 3

Explanation:
GVAbasic_prices=GDPmarket_prices+Subsidies on products-Taxes on products.

Q.6 Polity

Question:
Which of the following statements is correct regarding Fundamental Duties mentioned in the Indian Constitution?
I. The eleventh Fundamental Duty was added in 2002.
II. To protect the natural environment is a fundamental duty.

Options:
A) Both I and II
B) Neither I nor II
C) Only I
D) Only II

Answer: Option 1

Explanation:
Statement I: True. The 11th duty (education for children 6-14) was added by the 86th Amendment Act, 2002.
Statement II: True. Article 51A(g) mandates protecting and improving the natural environment.

Q.7 Census 2011

Question:
The percentage of Muslim in India as per Census 2011 is:

Options:
A) 11.6%
B) 15.6%
C) 13.2%
D) 14.2%

Answer: Option 4

Explanation:
According to the 2011 Census, the Muslim population in India was approximately 14.2%.

Q.8 Sports

Question:
[Karnataka] won 76th National football championship also known as Santosh Trophy in 2023.

Options:
A) Kerala
B) Manipur
C) Meghalaya
D) Karnataka

Answer: Option 4

Explanation:
Karnataka won the 2022-23 Santosh Trophy (76th edition) by defeating Meghalaya in the final held in Riyadh.

Q.9 Current Affairs (Environment)

Question:
In May 2023, scientists from the Zoological Survey of India (ZSI) discovered a new species of eel from Palur canal of _____.

Options:
A) Assam
B) Odisha
C) West Bengal
D) Tamil Nadu

Answer: Option 2

Explanation:
The new species, Pisodonophis kalinga, was discovered in the Palur canal in the Ganjam district of Odisha.

Q.10 Culture/Geography

Question:
Which Indian state is known as Dev Bhoomi due to having many pilgrimage places?

Options:
A) Uttarakhand
B) Punjab
C) Uttar Pradesh
D) Tamil Nadu

Answer: Option 1

Explanation:
Uttarakhand is famously referred to as "Dev Bhoomi" (Land of the Gods) because of its multitude of Hindu pilgrimage sites like Char Dham (Kedarnath, Badrinath, Gangotri, Yamunotri) and Haridwar.

Q.1 Vocabulary (Antonyms)

Question:
Choose the word that is opposite in meaning to the given word.
Disprove

Options:
A) Discuss
B) Express
C) Debunk
D) Substantiate

Answer: Option 4

Explanation:
Disprove: To prove that something is false.
Substantiate: To provide evidence to support or prove the truth of something.
Debunk is a synonym of disprove.

Q.2 One Word Substitution

Question:
Select the correct option to substitute the given set of words.
a short text which is added at the end of a book

Options:
A) Prologue
B) Epilogue
C) Epic
D) Monologue

Answer: Option 2

Explanation:
Epilogue: A section or speech at the end of a book or play that serves as a comment on or a conclusion to what has happened.
Prologue is the introduction.

Q.3 Grammar (Tense)

Question:
A sentence has been given with a blank to be filled with an appropriate option. Choose the correct alternative.
The little men ______ us rich, and we must show that we are grateful for it.

Options:
A) making
B) have made
C) does makes
D) had made

Answer: Option 2

Explanation:
The sentence uses the present tense "must show," implying the action's result is currently relevant. The Present Perfect Tense ("have made") is appropriate here to connect the past action (making them rich) with the present state of gratitude.

Q.4 Grammar (Error Spotting)

Question:
Which part of the sentence has an error?
She plans that she will got married by the end of 2028, if she gets a job of her choice.

Options:
A) by the end of 2028
B) that she will got married
C) if she gets a job
D) of her choice

Answer: Option 2

Explanation:
The error is in the phrase "will got married".
After the modal verb "will," the base form of the verb should be used.
Correct: "she will get married".

Q.5 Comprehension/Inference

Question:
Select the option that correctly infers the meaning of the given sentence.
The demand for the product is down, hence the low prices.

Options:
A) The price is down so the product is out of stock.
B) The demand is high which results in the low price of the product.
C) The price of the product is low because of the low market supply.
D) The product is low priced because there is not much demand in the market.

Answer: Option 4

Explanation:
The sentence connects low demand directly to low prices using "hence" (therefore). Option 4 correctly restates this cause-and-effect relationship: low demand → low price.

Q.6 Para Jumbles

Question:
Select the most logical order of the sentences to form a coherent paragraph.
P. Thus, that waste can be used to refill the pit.
Q. In many cases, a fairly large quantity of overburden or waste is excavated which does not contain many minerals.
R. Mining can alter any landscape drastically.
S. But it is costly to re-excavate it and refill the pit.

Options:
A) QRSP
B) QSRP
C) SPQR
D) RQPS

Answer: Option 4

Explanation:
R introduces the general topic (Mining altering landscape).
Q provides specific detail (Excavation creates waste/overburden).
P suggests a solution (Using waste to refill).
S presents a counter-point/challenge (It is costly).
Sequence: RQPS.

Q.7 One Word Substitution

Question:
Select the correct option to substitute the given set of words.
a person who knows many languages

Options:
A) Pragmatist
B) Polyglot
C) Grammarian
D) Linguist

Answer: Option 2

Explanation:
Polyglot: A person who knows and is able to use several languages.
Linguist is someone who studies the science of language (though often used for someone good at languages, Polyglot is the specific term for "knowing many").

Q.8 Para Jumbles

Question:
Select the most logical order of the sentences to form a coherent paragraph.
P. Human-caused methane is responsible for at least 25% of today's global warming.
Q. Methane is very powerful at trapping heat, making its global warming impact more than 80 times greater in the short term.
R. Around 60% of the world's methane emissions are produced by human activities.
S. Staunching those emissions, a new Global Methane Assessment by the United Nations Environmental Programme stresses, is the best hope for quickly putting the brakes on warming.

Options:
A) QSPR
B) RPSQ
C) PQRS
D) SQPR

Answer: Option 1

Explanation:
Q introduces the subject (Methane) and its power/impact.
S follows by discussing the solution ("Staunching those emissions") as the best hope.
P and R provide supporting statistics about the human role in these emissions.

Q.9 Grammar (Prepositions)

Question:
Fill in the blank with the correct option.
Elam has been working ______ the project with his team of experts.

Options:
A) to
B) with
C) on
D) for

Answer: Option 3

Explanation:
The correct phrasal verb/collocation is "working on" a project.

Q.10 Vocabulary

Question:
Choose the word that can substitute the given group of words.
To be a sign that something is going to happen soon

Options:
A) Premonition
B) Flog
C) Quiver
D) Bereave

Answer: Option 1

Explanation:
Premonition: A strong feeling that something is about to happen, especially something unpleasant. (While typically a noun, it is the only option fitting the definition of a "sign" of future events among the choices).